Amateur radio frequency bands

| Frequency bands in MHz | The Amateur Service | Maximum Power in dB relative to 1 watt PEP | Satellite | Permitted types of transmission | Remarks |
| --- | --- | --- | --- | --- | --- |
| 0.1357- 0.1378 | Secondary | CLASS A 20 dBW |  | All except pulse or fast scan TV |  |
| 0.472 — 0.479 | Secondary | CLASS A 20 dBW |  | All except pulse or fast scan TV |  |
| 1.810-2.000 | Primary | A = 30 dBW |  | All except pulse or fast scan 'IV | No Class B operation |
| 3.500 - 3.800 | Co-primary | - = 30 dBW - = 20 dBW |  | All except pulse or fast scan TV | Music Transmission permitted |
| 5.350 – 5.450 5,290 5kHz channel | Secondary primary | Class A 30 dBW Class B 20 dBW |  | All except pulse or fast scan TV |  |
| 7.000 - 7.200 | Primary | - = 30 dBW - = 20 dBW |  | All except pulse or fast scan TV |  |
| 10.100-10.150 |  | A = 26 dBW |  |  |  |

| 14.000 -14.350 | Primary | A = 30 dBW | Allocated | All except pulse or fast scan TV | No Class B operation International Band |
| --- | --- | --- | --- | --- | --- |
| 18.068 - 18.168 | Primary | A = 30 dBW |  | All except pulse or fast scan TV | No Class B operation International Band |
| 21.000 - 21.450 | Primary | A = 30 dBW | Allocated | All except pulse or fast scan TV | No Class B operation International Band |
| 24.890 - 24.990 |  | A =30dBW |  | All except pulse or fasl scan 'IN | No Class B International Band |
| 28.000 - 29.700 | Primary | A=30dBW | Allocated | All except pulse or fast scan TV | No Class B operation International Band |

| 28.050 -28.150 |  | B = 20 dBW |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 28.300 - 28.500 |  | B = 20 dBW |  |  |  |
| 29.700 - 30.000 | Secondary | 26 dBW |  |  | Secondary basis during disaster exercises and emergency |
| 40.675 - 40.685 | Primary | 26 dBW | Al located | All except pulse or fast scan TV | Propagation studies only |
| 50.000 - 54.000 | CO-Primary | 50-52 MHz = 30 dBW 53 -54MHz 26 dBW |  | All except pulse or fast scan TV | Music transmission permitted |
| 70.000- 70.300 | Secondary | Class A 26 dBW |  | All except pulse or fast scan |  |
| 144 — 146 | Primary | - = 30 dBW - = 20 dBW | Allocated | All except pulse or fast scan TV |  |
| 430 - 440 | CO-Primary | - 30dBW - 20 dBW | Allocated | All except pulse or fast scan TV | Music transmission permitted |
| 435 - 438 | Co-Primary | - = 30 dBW - = 20 dBW | Allocated | All except pulse or fast scan TV | Music transmission permitted. |

## 94 No. 38641	GOVERNMENT GAZETTE, 30 MARCH 2015

| 1240 - 1300 | Secondary | A = 30 dBW | Allocated | All except pulse | Music transmissions permitted |
| --- | --- | --- | --- | --- | --- |
| 2300 - 2450 | Secondary. Users must accept interference from ISM users | A = 26 dBW |  | All except pulse |  |
| 5650 - 5850 | Secondary. Users must accept interference from ISM users | A= 26 dBW | Allocated | All except pulse |  |
| 10000- 10450 | Secondary | A = 26 dBW | Allocated | All - including pulse |  |
| 10450 -10500 | Secondary | A = 26 dBW | Allocated | All - including pulse |  |
| 24000- 24050 | Primary | A = 26 dBW | Allocated | All - including pulse |  |
| 24050 -24250 | Secondary. Users must interference from ISM users | A = 26 dBW |  | Ali - including pulse |  |
| 47000 - 47200 | Primary | A = 26dBW | Allocated | All - including pulse |  |
| 75500 -76000 |  | A = 26 dBW | Allocated | All - including pulse |  |
| 76000 - 81000 | Secondary Users must | A = 26 dBW | Allocated | All - including pulse |  |

|  | interference from ISM users |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 122250 - 123000 | Secondary Users must accept interference from ISM users | A: 26 dBW | Allocated | All - including pulse |  |
| 134000 136000 |  | A: 26 dBW | Allocated | All — including pulse |  |
| 136000 141000 | Secondary | A = 26 dBW |  | All — including pulses |  |